OSISOS

Complete Technical Documentation & Build Roadmap

Resource-constrained Linux-based operating environment for PS2-class resource constraints

| Property | Target |
| --- | --- |
| Name | OSISOS |
| Core | Modular Linux-based operating environment |
| Language | C |
| Build | CMake + CTest |
| System builder | Buildroot |
| Emulation | QEMU |
| Resource envelope | ~32 MB RAM; ~300 MHz-class CPU; small storage |
| Interface | CLI-first |
| Networking | Core subsystem |
| Filesystem | Linux FS APIs; optional FUSE |
| Goal | Complete OS functionality under severe resource constraints |

# 1. Executive Summary

OSISOS is a modular operating environment built above the Linux kernel. Linux supplies low-level mechanisms—scheduling, virtual memory, networking, device drivers, filesystems and system calls. OSISOS supplies its own APIs, shell, process/resource management, filesystem management, IPC, concurrency, networking management, users/security, service management, logging, configuration and system utilities.

The defining constraint is resource efficiency. The project targets approximately 32 MB RAM, a single low-frequency CPU-class core and a small storage footprint. These are engineering targets to measure, not claims of actual PlayStation 2 hardware compatibility.

Core principle:

Linux provides mechanisms; OSISOS provides abstractions, policies, management and the user environment.

# 2. Objectives

Complete OS environment rather than unrelated scripts.

Process, memory, filesystem, IPC, concurrency, networking, security, service and system-management subsystems.

CLI-first operation with no required desktop environment.

PS2-class resource-envelope testing.

Bootable QEMU image.

Measured RAM, CPU, storage, boot-time and networking benchmarks.

Linux-specific details isolated behind wrapper/platform interfaces.

Portable core architecture suitable for future architectures.

# 3. Non-Objectives

Do not rewrite Linux's scheduler, VM system, TCP/IP stack or driver ecosystem.

Do not target actual PS2 hardware during the semester.

Do not make GUI functionality core.

Do not place Python/Node runtimes in the core image.

Do not begin with bootloader work.

# 4. Architecture

OSISOS
┌─────────────────────────────────────────────┐
│              OSISOS Applications            │
├─────────────────────────────────────────────┤
│                 OSISOS Shell                │
├─────────────────────────────────────────────┤
│              OSISOS Utilities               │
│ psx memx lsx net topx services users ...    │
├─────────────────────────────────────────────┤
│                 OSISOS Core                 │
│ Process │ Memory │ FS │ IPC │ Concurrency  │
│ Network │ Users  │ Services │ Resources    │
├─────────────────────────────────────────────┤
│               OSISOS API Layer              │
├─────────────────────────────────────────────┤
│             Linux Wrapper Layer             │
├─────────────────────────────────────────────┤
│              Minimal Linux Kernel           │
├─────────────────────────────────────────────┤
│                  Hardware                  │
└─────────────────────────────────────────────┘

High-level components should use OSISOS APIs. Linux-specific syscalls, /proc, /sys and platform details should be concentrated in wrapper/platform code.

# 5. Subsystem Specifications

## Process Management

Creation, termination, wait, process table, hierarchy, states, CPU/RAM statistics, process tree, priorities, background jobs and resource limits.

## Memory Management

System/per-process statistics, allocation tracking, memory pressure, limits and page-fault statistics. Linux remains responsible for virtual memory.

## Filesystem Management

File/directory operations, metadata, permissions, mounts, storage statistics, OSISOS filesystem API and optional FUSE/YOSFS.

## IPC

Pipes, FIFOs, message queues, shared memory, signals and Unix-domain sockets.

## Concurrency

Threads, mutexes, semaphores, condition variables, atomic operations, race/deadlock demonstrations.

## Networking

Interface discovery, static IP/DHCP, routes, DNS, TCP/UDP sockets, network status and tools.

## Shell Management

Lexer/parser/executor, pipelines, redirection, background jobs, history, environment variables, built-ins, scripts and signals.

## Service Management

Minimal init/supervisor, service definitions, dependencies, start/stop/restart/status, on-demand services and limits.

## Users & Security

Users, groups, authentication, permissions, privileged operations and service identities.

## System Management

sysinfo, monitor, logs, configuration, shutdown/reboot and benchmarks.

## Resource Management

Global RAM/CPU/storage accounting, service/process limits, memory-pressure detection and PS2-class benchmark profiles.

# 6. OSISOS API

osisos_process_create(); osisos_process_kill(); osisos_process_wait();
osisos_process_list(); osisos_process_info();

osisos_mem_stats(); osisos_mem_process_stats(); osisos_mem_limit_set();

osisos_fs_open(); osisos_fs_read(); osisos_fs_write();
osisos_fs_stat(); osisos_fs_mkdir(); osisos_fs_unlink();

osisos_ipc_pipe(); osisos_ipc_msg_send(); osisos_ipc_msg_recv();
osisos_ipc_shm_create();

osisos_thread_create(); osisos_mutex_lock(); osisos_mutex_unlock();
osisos_sem_wait(); osisos_sem_post();

osisos_net_if_list(); osisos_net_if_up(); osisos_net_if_down();
osisos_net_socket(); osisos_net_connect(); osisos_net_send(); osisos_net_recv();

osisos_user_create(); osisos_user_auth(); osisos_perm_check();

osisos_service_start(); osisos_service_stop(); osisos_service_status();

Define headers, structs, opaque handles and error codes before implementation. Prefer bounded/fixed-size data structures where resource predictability matters.

# 7. Shell

input → lexer → parser → command tree → executor → OSISOS API → Linux wrapper → kernel

osisos> psx
osisos> tree
osisos> memx
osisos> lsx
osisos> netinfo
osisos> pingx 192.168.1.1
osisos> services
osisos> users
osisos> topx
osisos> logs

# 8. Networking

Networking is a core requirement. Use Linux's networking stack rather than implementing TCP/IP.

OSISOS Network Manager
        │
 ┌──────┼──────────────┐
 ▼      ▼              ▼
Config  Sockets        Tools
 │      │              │
 └──────┼──────────────┘
        ▼
Linux socket/network APIs
        ▼
Linux TCP/IP stack
        ▼
Driver → NIC

IPv4, TCP and UDP.

Interface state and IP configuration.

DHCP and static configuration.

Routing and DNS.

pingx, netinfo, route, dnsx, tcpclient, tcpserver, udptest.

Networking between two OSISOS QEMU instances.

# 9. Filesystem

/
├── bin/
├── etc/
├── home/
├── proc/
├── sys/
├── tmp/
├── var/
└── osisos/

Start with ordinary Linux filesystem operations. Add FUSE only after the core API is stable. A future OSISOSFS can expose process/memory/system state as files.

# 10. Init and Service Management

PID 1: osisos-init
  ├── service manager
  ├── shell/session
  ├── logger
  └── optional services

service:
name
command
dependencies
restart_policy
uid/gid
memory_limit
cpu_limit
enabled

Prefer demand activation. Unused services should not consume resident RAM.

# 11. Resource Budget

| Metric | Initial target | Measurement |
| --- | --- | --- |
| RAM | 32 MB system envelope | QEMU + /proc + host measurement |
| CPU | 1 constrained vCPU | QEMU configuration + benchmark |
| Storage | 64–128 MB target image | Image-size measurement |
| Boot | <5 s target | init timestamp to shell-ready |
| Core processes | Minimal | Process inventory |
| Userspace overhead | Aim for single-digit MBs where feasible | RSS/system accounting |

Targets are hypotheses until measured. Record baseline, optimization and final values.

# 12. Toolchain and Build Strategy

Use CMake for OSISOS source development and CTest for automated tests. Use Buildroot for the final embedded Linux system. Buildroot can generate a cross-compilation toolchain, root filesystem, Linux kernel image and bootloader where applicable.

Developer:
OSISOS source → CMake → host binaries/tests

Target:
OSISOS source → cross compiler → rootfs
Linux kernel + rootfs → Buildroot image → QEMU

# 13. Development Resources

| Resource | URL | Purpose |
| --- | --- | --- |
| Linux Kernel Documentation | https://docs.kernel.org/ | Kernel and subsystem reference |
| Linux man-pages | https://man7.org/linux/man-pages/ | Syscalls, POSIX, sockets, processes, IPC |
| The Linux Programming Interface | https://man7.org/tlpi/ | Detailed Linux systems-programming reference |
| Buildroot Documentation | https://buildroot.org/docs.html | Embedded Linux image construction |
| Buildroot 2026.08 Manual | https://buildroot.buildroot.org/downloads/manual/manual.html | Current detailed Buildroot reference |
| QEMU System Docs | https://www.qemu.org/docs/master/system/ | Full-system emulation and configuration |
| CMake Docs | https://cmake.org/cmake/help/latest/ | Build system |
| CTest Tutorial | https://cmake.org/cmake/help/latest/guide/tutorial/Testing%20and%20CTest.html | Automated tests |
| libfuse API | https://libfuse.github.io/doxygen/ | Userspace filesystem |
| Linux FUSE docs | https://docs.kernel.org/filesystems/fuse/fuse.html | Kernel-side FUSE |
| BusyBox | https://busybox.net/ | Small embedded Unix utilities/reference |
| GNU Binutils | https://sourceware.org/binutils/ | Binary inspection and size analysis |

# 14. Development Environment

Linux host recommended.

GCC or Clang.

CMake + CTest.

Buildroot.

Linux kernel source/docs.

QEMU system emulation.

GDB.

binutils: size/readelf/objdump.

strace/perf for host-side diagnosis where available.

libfuse for optional filesystem milestone.

# 15. Repository Layout

osisos/
├── api/
├── core/{process,memory,filesystem,ipc,concurrency,network,users,services,resources}/
├── wrapper/{linux,procfs,sysfs}/
├── shell/
├── commands/
├── init/
├── logging/
├── config/
├── tests/{unit,integration,system}/
├── build/buildroot/
├── docs/
├── scripts/
├── CMakeLists.txt
└── README.md

# 16. Team of Five

| Member | Primary ownership | Secondary |
| --- | --- | --- |
| 1 | Process + resource management | Scheduler simulation, process tests |
| 2 | Filesystem + storage | OSISOSFS/FUSE, mount/metadata |
| 3 | IPC + concurrency | Threads, synchronization, IPC tests |
| 4 | Shell + system management | Parser, utilities, monitor, logging |
| 5 | Network + security + integration | Network manager, users, services, integration |

Ownership is not isolation: all members should understand the API and full architecture.

# 17. Semester Execution Plan

| Period | Milestone | Deliverable |
| --- | --- | --- |
| Week 1 | Architecture/API | Requirements, architecture, APIs, Git, CMake/test skeleton |
| Weeks 2–3 | Minimal OSISOS | Init prototype, shell, parser, sysinfo, first wrappers |
| Weeks 4–5 | Process + memory | Process table/tree, /proc, resource accounting |
| Weeks 6–7 | Filesystem | API, commands, metadata, mounts; optional FUSE |
| Weeks 8–9 | IPC + concurrency | Pipes, queues, shared memory, threads, synchronization |
| Week 10 | Networking | Interfaces, sockets, IP, DNS, TCP/UDP, two-instance test |
| Week 11 | Users + services | Permissions, users, minimal service manager |
| Week 12 | Optimization | Kernel/rootfs minimization, binary and RAM measurement |
| Week 13 | Bootable image | Buildroot + kernel/rootfs + QEMU |
| Week 14 | Integration | Stress, network, failure and API-freeze tests |
| Weeks 15–16 | Finalization | Benchmarks, report, presentation, reproducible build |

# 18. First 14 Days

Create repository and branch policy.

Write requirements.md and architecture.md.

Define API headers and error model.

Create CMake project and tests.

Implement shell loop.

Implement sysinfo.

Implement process listing from /proc.

Implement filesystem wrappers.

Complete first shell → API → Linux path.

Add process creation/termination.

Add memory statistics.

Add repeatable tests.

Create initial Buildroot configuration.

Boot a minimal image in QEMU and record baseline RAM/boot metrics.

# 19. Testing

## 19.1 Unit Tests

test_process_create
test_process_kill
test_fs_create
test_fs_read_write
test_ipc_message
test_mutex
test_socket
test_permission

## 19.2 Integration

shell → API → wrapper → Linux
network tool → socket API → Linux networking
service manager → process manager
resource manager → process/service limits

## 19.3 System Test

boot → shell → process → filesystem → IPC → network → service → shutdown

Use CTest to run the suite. CMake integrates testing through enable_testing()/add_test().

# 20. QEMU Test Matrix

| Profile | RAM | CPU | Purpose |
| --- | --- | --- | --- |
| Development | 512 MB+ | Fast/host-like | Feature development |
| Light | 256 MB | 1 vCPU | Optimization |
| PS2-class | 32 MB | 1 constrained vCPU | Primary target |
| Stress-low | 16 MB | 1 constrained vCPU | Find failure boundary |

QEMU system emulation provides a virtual CPU, memory and devices. Do not claim that its CPU constraint exactly reproduces a PS2 CPU; document the exact QEMU configuration used.

# 21. Buildroot Workflow

make menuconfig
  → select target architecture
  → minimal toolchain
  → Linux kernel
  → minimal utilities
  → networking features
  → OSISOS package
make
  → output/images/

Buildroot is particularly appropriate because it is designed to automate complete embedded Linux builds through cross-compilation and supports many architectures.

# 22. Kernel Strategy

Start from a known QEMU-compatible configuration.

Keep boot, storage, console and networking drivers required for the target.

Remove unused drivers/features only after a working baseline exists.

Keep debug features in development images; strip release artifacts where appropriate.

Avoid kernel modifications unless they have a clear project purpose.

Keep custom kernel patches/modules isolated and documented.

# 23. BusyBox Strategy

BusyBox is useful as a compact reference or base utility provider. OSISOS must not simply become a renamed BusyBox distribution: OSISOS-specific commands should demonstrate the OSISOS API and wrapper architecture.

# 24. Performance Optimization

Minimize dependencies.

Prefer C/small libraries for core components.

Build only required features.

Strip release binaries.

Measure binaries with size/readelf.

Avoid unnecessary background polling.

Use on-demand services.

Use bounded buffers.

Avoid unnecessary IPC/network copies.

Use bounded/circular logging.

Measure RSS and total system memory, not just binary size.

# 25. Security and Reliability

Use dedicated users for services where possible.

Centralize privileged operations.

Validate shell and network input.

Use Linux permissions/capabilities rather than inventing unnecessary parallel mechanisms.

Apply process/service resource limits.

Keep logs bounded.

Reap child processes.

Make optional service failure non-fatal.

Handle disk-full and low-memory conditions cleanly.

Prevent uncontrolled service restart loops.

# 26. MVP

OSISOS init and CLI shell.

Process creation/listing/termination.

Memory/system monitoring.

Filesystem management.

IPC and basic concurrency.

Networking with TCP/UDP and basic configuration/status.

Users/permissions.

Service management.

Resource accounting.

Automated tests.

Bootable Buildroot image.

Documented 32 MB benchmark.

# 27. Stretch Goals

OSISOSFS/FUSE.

Scheduler simulator.

Custom Linux syscall or small kernel module.

Resource-aware service activation.

Multiple CPU architectures.

IPv6.

Crash diagnostics.

Read-only immutable root filesystem.

Optional GUI image.

# 28. Final Demonstration

Boot OSISOS in QEMU under the 32 MB profile.

Show boot time and resource baseline.

Use sysinfo and resource profile.

Create/process-monitor several processes.

Demonstrate filesystem operations.

Demonstrate IPC between processes.

Demonstrate concurrent threads and synchronization.

Network two OSISOS instances and demonstrate TCP/UDP/ping.

Start/stop a service and show its resource use.

Demonstrate user/permission restrictions.

Run the benchmark and show RAM/CPU/storage/boot results.

# 29. Final Report Metrics

| Metric | Baseline | Optimized | Target/Observation |
| --- | --- | --- | --- |
| Boot time | TBD | TBD | <5 s target |
| Idle RAM | TBD | TBD | 32 MB envelope |
| Image size | TBD | TBD | 64–128 MB target |
| Idle processes | TBD | TBD | Minimal |
| Shell startup | TBD | TBD | Near-instant |
| Network throughput | TBD | TBD | Measured |
| TCP latency | TBD | TBD | Measured |
| Stable concurrent processes | TBD | TBD | Measured |
| Memory-pressure boundary | TBD | TBD | Measured |

# 30. Documentation Deliverables

requirements.md

architecture.md

api.md

build.md

qemu.md

networking.md

filesystem.md

security.md

resource-budget.md

testing.md

benchmark-results.md

troubleshooting.md

final-report.md

# 31. Engineering Rules

API first, implementation second.

Keep Linux-specific code behind wrapper/platform boundaries.

Measure every optimization.

Protect the MVP from stretch-feature creep.

Integrate continuously.

Test the 32 MB profile as soon as the first bootable image exists.

Keep the target CLI-first and daemon-light.

Use QEMU for reproducibility.

Use Buildroot for the final embedded image.

Record exact versions/configurations/commands.

Treat networking, concurrency, filesystem, shell and service management as first-class subsystems.

# 32. Authoritative Resources

The following resources were checked while preparing this document:

Buildroot manual: https://buildroot.buildroot.org/downloads/manual/manual.html

Buildroot docs: https://buildroot.org/docs.html

QEMU system documentation: https://www.qemu.org/docs/master/system/

QEMU introduction: https://www.qemu.org/docs/master/system/introduction.html

CMake/CTest testing: https://cmake.org/cmake/help/latest/guide/tutorial/Testing%20and%20CTest.html

libfuse API: https://libfuse.github.io/doxygen/

Linux FUSE documentation: https://docs.kernel.org/filesystems/fuse/fuse.html

Linux man-pages: https://man7.org/linux/man-pages/

BusyBox: https://busybox.net/

GNU Binutils: https://sourceware.org/binutils/

# 33. Final Project Definition

OSISOS is a modular, resource-constrained Linux-based operating environment that exposes core operating-system functionality through a unified abstraction layer. It combines process and memory management, filesystem management, IPC and concurrency, networking, shell management, service management, users/security, system management and resource accounting while deliberately targeting a PS2-class hardware resource envelope.